Introduction to Radar – HW3B

Reading: POMR 2nd ed., chapters 10, 11 and module 9.

Place MATLAB code at end of assignment in an appendix. You do not need to turn in the code.

- A radar needs a power amplifier for a transmitter with the following characteristics: 50 kW peak power, 5 kW average power, 10% bandwidth, 5 W drive power. What type of tube would be a suitable choice? Explain why.

- → A Klystron amplifier. It meets the high gain requirement of 40 dB (10log10(50000/5)), high power, and operates within the 1-10% bandwidth range.

- A MESFET microwave power transistor has a phase sensitivity of 10 deg/V of bias change. The DC power supply ripple voltage is 100 mV. Calculate the spurious sideband amplitude in dBc.

- →Δϕpeak=(10 deg/V)(100⋅10−3 V)=1∘

- →Δϕrad=1∘⋅π180∘≈0.01745 rad

- →dBc=20log10Δϕrad2=20log100.017452=−41.2 dBc

- What three external subsystems does the receiver interface with?

- → The Antenna, the Transmitter, and the Signal Processor.

- What is the total noise power produced from thermal noise of a component at room temperature with 1 GHz of instantaneous bandwidth?

- →PN=(−174 dBm/Hz)+10log10(1⋅109 Hz)=−174+90=−84 dBm

- A receiver front end undergoes a two-tone test to measure IP3. The input fundamentals are located at 8.3 and 8.31 GHz. Where do the third-order intermodulation signals appear?

- →fIM3,lower=2f1−f2=2(8.3 GHz)−8.31 GHz=8.29 GHz

- →fIM3,upper=2f2−f1=2(8.31 GHz)−8.3 GHz=8.32 GHz

- Sketch diagrams of the standard phased-array feed types. For each, give one pro and one con.

- → Constrained Series Feed:

- Pro: Simple structure. Con: Narrow bandwidth, beam squint.

- → Constrained Corporate Feed:

- Pro: Wide bandwidth. Con: Single point of failure.

- → Constrained Distributed Feed:

- Pro: Graceful degradation. Con: Increased complexity and cost.

- What is the purpose of anode and cathode in a linear beam tube?

- → The cathode emits electrons. The anode voltage accelerates the electrons to form a beam.

- The magnetron is what type of amplifier (three words)?

- →

- List three pros and three cons on a magnetron.

- → Pros: Inexpensive, Simple/Rugged, Relatively Lightweight.

- → Cons: Moding (frequency instability), Arcing/Missed Pulses (noisy), Non-coherent.

- Which type of amplifier has high-Q cavities?

- →

- List one pro and one con of a klystron amplifier.

- → Pro: High gain and high efficiency.

- → Con: Low (narrow) bandwidth.

- Referring to the figure, what parts of the radar system help control out-of-band and spurious emissions?

- → Primarily filters after the power amplifier. The modulator (pulse shaping) and amplifier linearity also contribute to minimizing their generation.

- What is the most common receiver type for MTI / Pulsed Doppler radars?

- →

- Define a duplexer.

- → A device that allows a single antenna to be used for both transmitting and receiving, protecting the receiver from the high-power transmitter.

- Define a circulator.

- → A passive, non-reciprocal three-port device that routes a signal from port 1 to port 2, from port 2 to port 3, and from port 3 to port 1.

- What are the differences between a circulator and a duplexer?

- → A duplexer is a general function (often a switch for pulsed radar), while a circulator is a specific component that provides this function by directing signal flow. Circulators are full-duplex capable but offer less isolation than a dedicated high-power switch.

- Discuss in a few sentences the concept of a channelized receiver.

- → A channelized receiver uses a bank of parallel filters to divide a wide operational bandwidth into many smaller, simultaneous channels. This allows for monitoring a wide spectrum at once.

- Find two commercial ADC converters that can sample at least 1 GHz and provide 9+ bits ENOB. Hint: See Table 11.1.

- → Analog Devices AD9208: 3 GSps, 9.7 ENOB

- → Texas Instruments ADC32RF45: 3 GSps, 10.2 ENOB

- Draw a sketch and discuss (2–3 sentences) the concept of OP1dB and IP1dB.

- → The 1 dB compression point (P1dB) marks the limit of an amplifier’s linear range. IP1dB is the input power that causes gain to drop by 1 dB, and OP1dB is the corresponding output power.

- An ADC has ENOB =8.3. What is the maximum SNR expected if the signal is near full scale?

- →SNR=(6.02⋅ENOB)+1.76=(6.02⋅8.3)+1.76=51.73 dB

- What are the primary two ways of sharing an antenna between TX and RX circuitry? Pros and cons of each?

- → T/R Switch: Pro: High isolation. Con: Half-duplex (pulsed systems only).

- → Circulator: Pro: Full-duplex capable. Con: Lower isolation.

- What is the purpose of sensitivity time control (STC)?

- → To prevent receiver saturation from strong close-range clutter by applying variable attenuation as a function of range.